FEMEX description

Every FE structural model will have the following data blocks:

- Geometry

- Materials

- Loads

- Boundary conditions

Geometry

To describe any geometry we need levels, nodes and elements.

Level is simply a Z-coordinate with some additional properties, like elevation above zero (reduced level, RL), isGround for ground levels, etc.

{

"$schema": "https://json-schema.org/draft/2020-12/schema",

"title": "LevelDefinition",

"description": "Definition of a building level including its vertical positioning and ground status.",

"type": "object",

"properties": {

"level_number": {

"type": "integer",

"description": "The sequential index of the level (e.g., 0 for ground, 1 for first floor)."

},

"absolute_elevation": {

"type": "number",

"description": "The elevation relative to a global datum like sea level."

},

"relative_elevation": {

"type": "number",

"description": "The elevation relative to the project internal zero point."

},

"isGround": {

"type": "boolean",

"description": "Flag indicating if this level is considered the primary ground plane."

}

},

"required": ["level_number", "absolute_elevation", "relative_elevation", "isGround"],

"additionalProperties": false

}

{

"levels": [

{

"level_number": 0,

"absolute_elevation": 145.50,

"relative_elevation": 0.00,

"isGround": true

},

{

"level_number": 1,

"absolute_elevation": 148.50,

"relative_elevation": 3.00,

"isGround": false

}

],

"points": [

{

"node_id": 1,

"x": 10.0,

"y": 5.0,

"level": 1,

"vertical_offset": 0.2

}

]

}

Nodes should have X and Y coordinates, Level property, and vertical offset from Level, thus defining Z coordinate.

{

"$schema": "https://json-schema.org/draft/2020-12/schema",

"title": "LevelBasedPointCloud",

"description": "A point cloud where vertical position is defined by a level index and an offset.",

"type": "object",

"properties": {

"points": {

"type": "array",

"items": {

"type": "object",

"properties": {

"node_id": {

"type": "integer"

},

"x": { "type": "number" },

"y": { "type": "number" },

"level": {

"type": "integer",

"description": "The floor or reference level number."

},

"vertical_offset": {

"type": "number",

"description": "The vertical distance from the specified level."

}

},

"required": ["node_id", "x", "y", "level", "vertical_offset"],

"additionalProperties": false

}

}

},

"required": ["points"]

}

{

"points": [

{

"node_id": 101,

"x": 12.5, "y": 8.2,

"level": 2,

"vertical_offset": 0.45

},

{

"node_id": 102,

"x": 14.0,

"y": 7.1,

"level": 3,

"vertical_offset": -0.10

}

]

}

Elements can be bars or plates. Bars are defined by start and end nodes. Plates are defined by the sequence of node numbers, node sequence matters.

Element sections are part of geometry. Bar sections (rectangular, circular, T-shape, etc.) are stored separately. Each bar should have a section, and angle of rotation of local X-axis relative to global X-axis. Plate thickness is a property of a plate and is a single number.

Every element has local axes.

Materials

As a bare minimum, a material should have the following parameters:

- modulus of elasticity E

- Poisson's ratio

- unit weight

- potentially, strength (not needed for analysis, but necessary for design)

Loads

Concentrated loads: forces and moments.

Linearly distributed loads: forces and moments.

Area loads.

Temperature.

Boundary conditions

Supports

- point, linear, or area

- degrees of freedom

- infinite or finite stiffness

Hinges

- point or linear

- degrees of freedom

- full or partial

- belong to element ends or edges